Networking Training Program (Module 6)

1. ARP

- ARP resolves IP addresses to MAC addresses, enabling devices to communicate within a local network.

- It allows routers to determine the next-hop MAC address for packet forwarding.

- ARP caching improves efficiency by reducing repeated address lookups.

- Without ARP, network devices wouldn’t know where to send packets, disrupting communication.

Screenshot:

Find the Ip to ping using ipconfig

Ping the ip to generate traffic :

ARP request and response in Wireshark:

2. Manually configure and Traceroute Tracking

PING TO THE PC3 IT REACHES AFTER SETTING THE STATIC ROUTING :

Using tracert :

3. Network Subnetting

Subnetting 10.0.0.0/24 into 4 Equal Subnets

1. Given Network Information

- Network Address: 10.0.0.0/24

- Subnet Mask: 255.255.255.0

- Total Hosts in /24: 2⁸ - 2 = 254 hosts

- Requirement: Divide into 4 equal subnets

2. Calculate the New Subnet Mask

- To divide into 4 subnets, we need 2 extra bits (2² = 4).

- The new subnet mask becomes /26 (255.255.255.192).

- Each subnet has 2⁶ - 2 = 62 usable hosts.

3. Subnet Allocation and Host Ranges

| Subnet | Network Address | First Usable IP | Last Usable IP | Broadcast Address |
| --- | --- | --- | --- | --- |
| 1 | 10.0.0.0/26 | 10.0.0.1 | 10.0.0.62 | 10.0.0.63 |
| 2 | 10.0.0.64/26 | 10.0.0.65 | 10.0.0.126 | 10.0.0.127 |
| 3 | 10.0.0.128/26 | 10.0.0.129 | 10.0.0.190 | 10.0.0.191 |
| 4 | 10.0.0.192/26 | 10.0.0.193 | 10.0.0.254 | 10.0.0.255 |

4. Assign IP Addresses to Devices in Packet Tracer

- Router (Default Gateway): Assign first usable IP from each subnet.

- PCs, Switches: Assign IPs within the valid host range.

Example Device IP Assignments

| Device | Assigned IP | Subnet Mask | Gateway |
| --- | --- | --- | --- |
| Router | 10.0.0.1 | 255.255.255.192 | N/A |
| PC1 | 10.0.0.2 | 255.255.255.192 | 10.0.0.1 |
| PC2 | 10.0.0.65 | 255.255.255.192 | 10.0.0.64 |
| PC3 | 10.0.0.129 | 255.255.255.192 | 10.0.0.128 |
| PC4 | 10.0.0.193 | 255.255.255.192 | 10.0.0.192 |

5. Verify Connectivity

- Ping Test:

  - Use the ping command to check connectivity between devices in the same and different subnets.

  - Example:

ping 10.0.0.65

- Traceroute Test:

  - Use tracert or traceroute to check packet paths between different subnets.

  - Example:

tracert 10.0.0.129

- Check Routing Table (On Router):

show ip route

4. IP Address Classification and NAT Explanation

1. Identifying the Class of Each IP Address

IP addresses are categorized into different classes based on their first octet.

| IP Address | First Octet Range | Class |
| --- | --- | --- |
| 192.168.10.5 | 192 - 223 | Class C |
| 172.20.15.1 | 128 - 191 | Class B |
| 8.8.8.8 | 1 - 126 | Class A |

2. Determining if the IP is Private or Public

Some IP ranges are reserved for private networks, while others are public and routable on the internet.

| IP Address | Private/Public | Reason |
| --- | --- | --- |
| 192.168.10.5 | Private | Falls in 192.168.0.0 - 192.168.255.255 (Private Class C) |
| 172.20.15.1 | Private | Falls in 172.16.0.0 - 172.31.255.255 (Private Class B) |
| 8.8.8.8 | Public | Google DNS, routable on the internet |

5. NAT Translation

- NAT translates private IP addresses into a public IP address.

- The router replaces the source IP of outgoing packets.

- Inside the network, packets use private IP addresses.

- Outside the network, packets use the router’s public IP.

- This allows multiple devices to share a single public IP.

- Wireshark captures show different source IPs before and after NAT.